Supah_0.9873

Model coordinates · Coordinate del modello · Sundai @ MIT, Track 2 · October 18, 2026

# Italiano

Ecco le coordinate di input che danno 0.9873: è il modello Supah_0.9873, punteggio ufficiale sulla griglia 64 × 64 (0.9891 sulla griglia finale 128 × 128).

{

"placements_mm": {

"G0": [4.869, 3.289],

"H0": [18.619, 1.773],

"G1": [32.573, 3.089],

"H1": [31.232, 15.059],

"G2": [31.122, 27.751],

"H2": [18.619, 29.757],

"G3": [5.356, 27.792],

"H3": [5.36, 14.975]

}

}

| Die | x (mm) | y (mm) | Dimensioni (mm) | Tipo |
| --- | --- | --- | --- | --- |
| G0 | 4.869 | 3.289 | 11.5 × 10.5 | GPU |
| H0 | 18.619 | 1.773 | 10.5 × 10.2 | HBM |
| G1 | 32.573 | 3.089 | 11.5 × 10.5 | GPU |
| H1 | 31.232 | 15.059 | 10.5 × 10.2 | HBM |
| G2 | 31.122 | 27.751 | 11.5 × 10.5 | GPU |
| H2 | 18.619 | 29.757 | 10.5 × 10.2 | HBM |
| G3 | 5.356 | 27.792 | 11.5 × 10.5 | GPU |
| H3 | 5.360 | 14.975 | 10.5 × 10.2 | HBM |

## Come leggerle

- Sono in millimetri e indicano l'angolo in basso a sinistra di ogni die. L'origine è l'angolo in basso a sinistra dell'interposer (52 × 42 mm).

- Dimensioni: GPU (G0–G3) 11.5 × 10.5 mm, HBM (H0–H3) 10.5 × 10.2 mm. Nessuna rotazione.

- Non ci sono wire_routes_grid: i collegamenti usano il router automatico dello scorer.

## Attenzione se le usi come punto di partenza

- Il layout usa 13,435.5 su 13,500 GB/s·mm di budget dei fili. Quasi ogni spostamento che allunga un filo lo rende illegale, quindi conviene controllare le regole a ogni mossa con verify_submission.py.

- Il punteggio dipende solo da G0 nel workload skewed_compute (103.58 °C). Il caso subito dopo, G1 in tensor_training (101.82 °C), ha circa 1.8 °C di margine, e la HBM ha circa il 2% di margine. Le mosse utili sono quelle che raffreddano G0.

Il file è nel repo del team: Models/Supah_0.9873/submission.json, con i report ufficiali, il floorplan e la heat map nella stessa cartella. Una copia è anche in model_coordinates/.

# English

These are the input coordinates that score 0.9873: model Supah_0.9873, official score on the 64 × 64 grid (0.9891 on the 128 × 128 finals grid).

{

"placements_mm": {

"G0": [4.869, 3.289],

"H0": [18.619, 1.773],

"G1": [32.573, 3.089],

"H1": [31.232, 15.059],

"G2": [31.122, 27.751],

"H2": [18.619, 29.757],

"G3": [5.356, 27.792],

"H3": [5.36, 14.975]

}

}

| Die | x (mm) | y (mm) | Size (mm) | Type |
| --- | --- | --- | --- | --- |
| G0 | 4.869 | 3.289 | 11.5 × 10.5 | GPU |
| H0 | 18.619 | 1.773 | 10.5 × 10.2 | HBM |
| G1 | 32.573 | 3.089 | 11.5 × 10.5 | GPU |
| H1 | 31.232 | 15.059 | 10.5 × 10.2 | HBM |
| G2 | 31.122 | 27.751 | 11.5 × 10.5 | GPU |
| H2 | 18.619 | 29.757 | 10.5 × 10.2 | HBM |
| G3 | 5.356 | 27.792 | 11.5 × 10.5 | GPU |
| H3 | 5.360 | 14.975 | 10.5 × 10.2 | HBM |

## How to read them

- Millimetres, giving the lower-left corner of each die. The origin is the lower-left corner of the 52 × 42 mm interposer.

- Sizes: GPUs (G0–G3) 11.5 × 10.5 mm, HBM stacks (H0–H3) 10.5 × 10.2 mm. No rotation.

- No wire_routes_grid: links use the scorer's automatic router.

## If you use them as a starting point

- The layout uses 13,435.5 of the 13,500 GB/s·mm wire budget. Almost any move that lengthens a wire makes it illegal, so check the rules after every move with verify_submission.py.

- The score depends only on G0 in the skewed_compute workload (103.58 °C). The next case, G1 in tensor_training (101.82 °C), has about 1.8 °C to spare, and the HBM has about 2% to spare. Useful moves are the ones that cool G0.

The file is in the team repo at Models/Supah_0.9873/submission.json, next to its official reports, floorplan and heat map. A copy is also in model_coordinates/.

## For comparison / Per confronto

| Die | Reference (1.000) | Supah_0.9987 | Supah_0.9873 |
| --- | --- | --- | --- |
| G0 | 7.563, 3.259 | 6.001, 2.455 | 4.869, 3.289 |
| H0 | 20.934, 3.584 | 18.861, 1.664 | 18.619, 1.773 |
| G1 | 33.272, 2.856 | 33.248, 2.485 | 32.573, 3.089 |
| H1 | 34.120, 15.888 | 32.413, 14.627 | 31.232, 15.059 |
| G2 | 33.072, 27.366 | 31.240, 27.052 | 31.122, 27.751 |
| H2 | 20.770, 27.311 | 19.646, 30.013 | 18.619, 29.757 |
| G3 | 7.994, 27.611 | 4.892, 29.010 | 5.356, 27.792 |
| H3 | 8.748, 15.163 | 5.777, 15.179 | 5.360, 14.975 |

Each cell is x, y in mm (lower-left corner). / Ogni cella è x, y in mm (angolo in basso a sinistra).